- FTP - File Transfer Protocol

  - tcp/20 (active mode data)

  - tcp/21(control)

- Transfers files between systems

- SSH - Secure Shell

  - tcp/22

- Encrypted communication link

-Looks and acts the same as Telnet

- Telnet-

  - tcp/23

- Non encrypted communication console access

- Not the best choice to use most people wont

- SMTP - Simple Mail Transfer Protocol

  - tcp/25

- Sending mail. Server to server email transfer

- IMAP4 - Internet Message Access Protocol v4

- Tcp143

- tcp/993 imap over ssl

- POP3 - Post office Protocol version 3

  - tcp/110

  - tcp/995 POP3 over SSL

- DNS - Domain Name System

  - udp/53

- HTTP & HTTPS - HyperText Transfer Protocol/Secure

  - tcp/80 HTTP - Web Server Communication

  - tcp/443 HTTPS - Web Server Communication with encryption

- RDP - Remote Desktop Protocol

  - tcp/3389

  - Share a desktop from a remote location, Remote Desktop services on many window versions, Can connect to an entire desktop or just an application. Client for Windows macOS, Linux, Unix, iPhone, Android, and others.

- SMB - Server Message Block-tcp/445

  - File sharing, printer sharing

  - Also called CIFS (Common Internet File System)

- AFP - Apple Filing Protocol

  - File services in macOS tcp/548

- DHCP - Dynamic Host Configuration Protocol

  - udp/67, udp/68

  - Automated configuration of IP address, subnet mask and other options

  - Requires a DHCP server which is a server appliance that is usually integrated in a SOHO router etc. Server can recognize MAC addresses

  - Uses DORA process to assign IP addresses

- LDAP - Lightweight Directory Access Protocol

  - tcp/389

  - Store and retrieve information in a network directory/Commonly used in Microsoft Active Directory

- SNMP - Simple Network Management Protocol

  - udp/161 for Queries

  - udp/162 Traps

  - Receive information about the network etc.

- NTP -

  - tcp/123

- Net bios is tcp/137